Based on the provided presentation slides, here is the structured, step-by-step solution to find the adiabatic flame temperature (Tad) for the combustion of carbon monoxide (CO).

## Step 1: Establish the Basis and Chemical Equation

First, we define our calculation basis and write down the balanced stoichiometric chemical reaction for the complete combustion of CO:

- Basis: 1 mol of CO

- Stoichiometric Reaction:

CO+12O2→CO2

## Step 2: Perform the Mass Balance (Determine Product Moles)

The fuel is burned with 120% excess air. This means the total air supplied is 2.2 times (1+1.2) the theoretical amount required.

- Theoretical Oxygen (O2) Required: For 1 mol of CO, we need 0.5 mol of O2.

- Actual Air Supplied: Since air is roughly 21% O2 and 79% N2:

Theoretical Air=0.5 mol O20.21=2.381 mol air

Actual Air Fed=2.2×2.381 mol=5.238 mol air

- Component Breakdowns:

- N2 entering (and exiting unchanged): 0.79×5.238 mol=4.138 mol

- O2 entering: 0.21×5.238 mol=1.1 mol

- Excess O2 exiting: O2 in−O2 consumed=1.1−0.5=0.6 mol

- CO2 produced: 1 mol (from 1 mol of CO)

### Summary of Output Product Moles (ni):

- nCO2=1.0 mol

- nN2=4.138 mol

- nO2=0.6 mol

## Step 3: Formulate the Energy Balance

Under an adiabatic condition, no heat is lost to the surroundings (Q=0). Because both the reactants (fuel and air) enter at the standard reference temperature of 25∘C , their sensible enthalpy is zero (ΔHreactants=0).

Therefore, the standard heat of reaction (ΔHr∘) released by the combustion must entirely go into raising the temperature of the product gases from 25∘C to Tad:

−ΔHr,25∘C∘=∑ni⋅Hi(Tad)

Where:

- ΔHr,25∘C∘ for CO+12O2→CO2 is calculated using standard heats of formation:

ΔHr∘=ΔHf∘(CO2)−ΔHf∘(CO)=−393.51−(−110.53)=−282.98 kJ/mol

- This gives the target total enthalpy for the products at Tad:

∑ni⋅Hi(Tad)=282.98 kJ

## Step 4: Trial & Error Using Specific Enthalpy Tables

To find the exact value of Tad, look at Table B.8 (Specific Enthalpies of Selected Gases) to calculate the total product enthalpy (∑niHi) at guessed temperatures until it equals 282.98 kJ:

### Trial 1: Guess T=1400∘C

From Table B.8:

- HCO2=71.89 kJ/mol

- HN2=44.51 kJ/mol

- HO2=47.07 kJ/mol

∑niHi=(1.0)(71.89)+(4.138)(44.51)+(0.6)(47.07)=71.89+184.18+28.24=284.31 kJ

(This value is slightly higher than our target of 282.98 kJ, meaning Tad is just below 1400∘C.)

### Trial 2: Guess T=1300∘C

From Table B.8:

- HCO2=65.98 kJ/mol

- HN2=40.97 kJ/mol

- HO2=43.38 kJ/mol

∑niHi=(1.0)(65.98)+(4.138)(40.97)+(0.6)(43.38)=65.98+169.53+26.03=261.54 kJ

(This value is lower than our target of 282.98 kJ.)

## Step 5: Linear Interpolation

Since the target value 282.98 kJ lies between 261.54 kJ (at 1300∘C) and 284.31 kJ (at 1400∘C), apply linear interpolation to isolate the exact temperature:

Tad=1300+(1400−1300)×282.98−261.54284.31−261.54

Tad=1300+100×21.4422.77

Tad≈1394∘C

Final Assessment: The maximum temperature the reactor walls must be designed to withstand is approximately 1394∘C.