Faculty of Chemical and Energy Engineering

Universiti Teknologi Malaysia

| SETN 3224 Thermal Hydraulics and Lab | Semester I 2025-2026 |
| --- | --- |
| Lab 7 – PCTRAN - Small-Break LOCA (SBLOCA) Transient in a Two-Loop PWR | Due: 30 December 2025 |

Objectives:

- Trigger and analyze a cold-leg SBLOCA in a two-loop PWR model.

- Record key temperatures and flow rates revealing primary/secondary heat removal and ECCS performance.

- Compare outcomes across break sizes and key system variations.

Introduction:

In this computer-based lab, you will use the PCTRAN two-loop PWR simulator to investigate the plant response to a Small-Break Loss-of-Coolant Accident (SBLOCA) initiated on a cold leg. Starting from full-power steady state, you will trigger a range of break sizes and operate key safety systems (AFW, HPSI/ACC/LPSI) to observe how primary and secondary inventories, temperatures, and flows evolve during depressurization and recovery. By recording required temperature and flow-rate plots—along with event timestamps—you will analyze subcooling margin, heat removal balance across the steam generators, and the effectiveness of different ECCS lineups. The exercise builds practical intuition for RPS/SFAS actuation logic and highlights which operator actions most strongly influence core cooling and transient stabilization.

Simulation Procedures

## Initial Condition (All Runs)

Power: 100% rated; automatic controls normal; all RCPs running; AFW available but off; pressurizer mid-level.

Start from the standard 100% power initial condition (IC).

## Transient Definition

Malfunction: Cold-leg break (SBLOCA).

Break areas (run all three): Case A = 5 cm², Case B = 20 cm², Case C = 60 cm².

Choose ONE variation to pair with your chosen break size:

AFW start delay = 0 s vs 180 s, or

Pressurizer initial level = 45% vs 65%, or

ECCS lineup: HPSI both trains vs one-train-out (accumulators and LPSI available).

## Required Plots (Flow Rates & Temperatures)

Create one figure per bullet (multi-curve), with units and legends. Time on x-axis.

### 1) Primary-side Temperatures & Pressure

TAVG; Hot-leg A/B (THA/THB); Cold-leg A/B (TCA/TCB); RCS Pressure (overlay).

- Comment: Hot/cold leg split, depressurization slope, approach to saturation.

### 2) Primary-side Flows (Core Cooling Picture)

RCS loop flows A & B (WRCA, WRCB); Break flow (WLR); ECCS flows: HPSI (WHPI), LPSI (WLPI), Total ECCS (WECS).

- Comment: Timing of HPI/LPI relative to pressure; refill/reflood trends; effect of break size.

### 3) Secondary Heat-Removal Balance

SG feedwater flows A & B (WFWA, WFWB); SG steam flows A & B (WSTA, WSTB); SG pressures A & B (PSGA, PSGB) or steam header pressure.

- Comment: AFW timing vs SG level/pressure; steam removal sufficiency.

### 4) Inventory Indicators (Compact Panel)

Pressurizer level (LVPZ); RCS liquid volume (VOL); PORV/safety discharge flow (WUP).

- Comment: Early-time PRZ behavior; any valve lifts; inventory loss vs recovery.

### 5) Thermal Margin Tracking

Subcooling margin (SCM-A, SCM-B) or plot (TAVG vs Tsat) if subcooling tag not available; Pressurizer/loop Tsat (TSAT).

- Comment: Timestamp of minimum margin and recovery; which systems drove recovery.

## Step-by-Step (Per Run)

Load 100% power IC; verify autos (rods, PRZ, SG, turbine).

Start transient (Run).

Inject cold-leg break; set area (e.g., 20 cm²) and Activate.

Operate your chosen variation (e.g., start AFW at 180 s; disable one HPSI pump).

Record timestamps for: reactor trip, HPSI start, accumulator open, LPSI start (if used), first AFW start, any containment pressure alarm.

Save all five plot sets above (with units/legends).

Capture P/T diagram screenshots at (i) minimum subcooling and (ii) after recovery.

## Sequence-of-Events Table (Fill In)

| Event | Time (s) |
| --- | --- |
| Reactor Trip (RPS) |  |
| HPSI starts |  |
| Accumulators open |  |
| LPSI starts (if applicable) |  |
| AFW starts (your delay) |  |
| Min subcooling time |  |
| Subcooling recovered |  |

## Plot Templates (Tag Lists to Select in PCTRAN)

Primary Temperatures & Pressure: TAVG, THA, THB, TCA, TCB, RCS_PRESS

Primary Flows: WRCA, WRCB, WLR, WHPI, WLPI, WECS

Secondary Heat Removal: WFWA, WFWB, WSTA, WSTB, PSGA, PSGB (or Steam Header Pressure)

Inventory & Relief: LVPZ, VOL, WUP

Thermal Margin: SCMA, SCMB, TSAT (or TAVG vs TSAT where available)

Discussion prompts

- How did break area change depressurization, ECCS start order/timing, and minimum subcooling?

- How did your variation (AFW delay / PRZ level / ECCS lineup) affect SG heat removal, refill, and margin recovery?

- Which single system action had the largest impact on avoiding loss of subcooling?

Deliverables

Your report should include the background description of the experiment, objectives, procedures, data collected, results calculated, discussions, conclusion, and references. You should also include the video and photos of the relevant moments of the experiment. Hand in your work in a zip file containing all relevant files.

| Acronym | Meaning |
| --- | --- |
| PWR | Pressurized Water Reactor |
| SBLOCA | Small-Break Loss of Coolant Accident |
| ECCS | Emergency Core Cooling System |
| HPSI (HPI) | High-Pressure Safety Injection (High-Pressure Injection) |
| LPSI (LPI/RHR) | Low-Pressure Safety Injection (Residual Heat Removal) |
| ACC | Accumulator (nitrogen-pressurized ECCS injection tank) |
| AFW | Auxiliary Feedwater |
| RCS | Reactor Coolant System |
| RCP | Reactor Coolant Pump |
| PRZ | Pressurizer |
| PORV | Power-Operated Relief Valve (pressurizer relief) |
| SG | Steam Generator |
| RB | Reactor Building (containment) |
| PCT | Peak Cladding Temperature |
| RPS | Reactor Protection System (trips/scram) |
| SFAS | Safety Features Actuation System |
| OTΔT / OPΔT | Over-Temperature / Over-Power Delta-T protection setpoints |
| TAVG | RCS average temperature |
| THA / THB | Hot-leg temperature, Loop A / Loop B |
| TCA / TCB | Cold-leg temperature, Loop A / Loop B |
| TSAT | Saturation temperature (at system pressure) |
| SCM / SCMA / SCMB | Subcooling margin (overall / Loop A / Loop B) |
| WRCA / WRCB | RCS mass flow, Loop A / Loop B |
| WFWA / WFWB | Feedwater flow, SG A / SG B |
| WSTA / WSTB | Steam flow, SG A / SG B |
| WLR | Leak (break) flow rate |
| WUP | Pressurizer relief/safety discharge flow |
| WHPI / WLPI / WECS | HPSI flow / LPSI flow / Total ECCS flow |
| PSGA / PSGB | Steam generator pressure, A / B |
| LVPZ | Pressurizer level |
| VOL | RCS liquid volume (inventory indicator) |
| P/T diagram | Pressure–Temperature diagram (used to read subcooling) |
| NPSH | Net Positive Suction Head (pump suction margin) |
| IC | Initial Condition (simulator starting state) |
| U-tube | Steam generator heat-exchange tube geometry in PWRs |
| PCTRAN | Personal Computer Transient Analyzer (the simulator) |